WUALT

Geospatial Context &

Safety Risk Model

Technical Architecture, Rule Engine Logic, Risk Scoring System,
and Implementation Guide

| Property | Value |
| --- | --- |
| Document Version | 2.0 |
| Date | April 2026 |
| Classification | Confidential — Internal Engineering |
| System | WUALT Smart Ring — Safety Intelligence Engine |
| Architecture | Rule-Based / Interpretable (No ML) |
| Latency Target | < 700 ms per evaluation |
| Risk Levels | 5 (normal → low → moderate → high → critical) |
| Detection Layers | 3 (Physiological + Fall + Geospatial) |

# 1. Executive Summary

The WUALT Geospatial Safety Risk Model extends the existing physiological distress detection engine by incorporating location, movement, time, and familiarity context. The system answers a single question:

Core Question "Given the user's body signals and current context, how concerning is the situation right now?"

The model combines two independent signal streams — physiological state (heart rate, SpO2, HRV, temperature) and geospatial context (GPS location, time of day, movement patterns, zone familiarity) — to produce a unified 5-level safety risk classification with explainable reasoning and calm, actionable alerts.

Key design principles:

- Context is a modifier, never a trigger — geospatial data amplifies or suppresses physiological signals

- Every decision is traceable — the reasoning chain shows exactly why a risk level was assigned

- Privacy-first — raw GPS coordinates are never stored, only zone classifications

- Women’s safety focus — escalation patterns tuned for real-world threat scenarios

- Low false positives — suppression rules prevent alerts during exercise, at home, or at work

# 2. System Architecture

The unified safety engine operates as a three-layer detection stack:

## 2.1 Layer 1: Physiological Distress Detection

The existing distress engine processes raw wearable sensor data through a seven-stage pipeline: SQI gate, motion classification, signal flagging (z-score + absolute thresholds), motion-aware suppression, state determination, persistence tracking, and alert selection.

Outputs one of three states: normal, stress, or distress — with a confidence score (0–1) and a list of contributing signals.

## 2.2 Layer 2: Fall Detection

A three-stage accelerometer-based fall detector:

- Stage 1 — Impact Detection: Free-fall (acc_mag < 0.5g) followed by impact spike (> 3.0g) within a 5-second window

- Stage 2 — Orientation Change: Body angle shift > 60° from pre-fall vertical, estimated from accelerometer axis ratios

- Stage 3 — Post-Fall Inactivity: Stillness (dyn_acc < 0.05g) sustained for 3+ seconds confirms the fall; movement resumes → false alarm

## 2.3 Layer 3: Geospatial Safety Risk (New)

The geospatial layer takes the physiological output and combines it with context to produce a unified risk assessment. It does NOT detect anything independently — it modifies the physiological signal based on how concerning the current context is.

Architecture flow:

- Score each context dimension independently (time, location, movement, connectivity, familiarity)

- Compute weighted context score (0–1)

- Multiply physiological base risk by context amplifier (0.8x–1.5x)

- Apply suppression rules for safe contexts (home, gym, work)

- Apply escalation patterns for dangerous combinations

- Map final score to 5-level risk classification

- Select context-appropriate alert message

## 2.4 Unified Engine (Orchestrator)

The UnifiedSafetyEngine class orchestrates all three layers in a single evaluate() call. It runs physiological detection first, then fall detection, then geospatial risk assessment, and finally selects the highest-priority alert across all layers.

Design Principle The geospatial context NEVER triggers alerts by itself. Even at 2 AM in an unfamiliar area with no phone, if the user’s body signals are normal, the risk stays low. Context only matters when the body is also showing concern.

# 3. Input Specification

## 3.1 Physiological Inputs (from Distress Engine)

| Field | Type | Description |
| --- | --- | --- |
| state | string | "normal" ¦ "stress" ¦ "distress" |
| confidence | float | Detection confidence (0.0 – 1.0) |
| contributing_signals | list | Which signals triggered: hr, hrv, spo2, temp |

## 3.2 Geospatial Context Inputs (from Phone GPS)

All fields have safe defaults so the system gracefully degrades when partial context is available.

| Field | Type | Default | Description |
| --- | --- | --- | --- |
| latitude | float | 0.0 | GPS latitude (used for distance calc only, never stored) |
| longitude | float | 0.0 | GPS longitude (used for distance calc only, never stored) |
| timestamp | int | 0 | Unix timestamp of the reading |
| speed_kmph | float | 0.0 | Current speed in km/h |
| heading | float | 0.0 | Direction of travel in degrees |
| is_home_zone | bool | false | User is within their defined home geofence |
| is_work_zone | bool | false | User is within their defined work geofence |
| is_known_area | bool | true | Area has been visited before (default safe) |
| is_unfamiliar_area | bool | false | Area has never been visited |
| distance_from_home_km | float | 0.0 | Straight-line distance from home |
| hour_of_day | int | 12 | Current hour (0–23) |
| is_night | bool | false | Whether it is nighttime (20:00–05:59) |
| is_stationary | bool | true | User is not moving |
| is_walking | bool | false | User is walking (GPS speed 2–7 km/h) |
| is_vehicle_like_motion | bool | false | Speed suggests vehicle (> 15 km/h) |
| sudden_route_change | bool | false | Heading changed > 90° unexpectedly |
| sudden_stop | bool | false | Speed dropped from > 20 to < 2 km/h rapidly |
| phone_connected | bool | true | Ring has active BLE connection to phone |

# 4. Output Specification

The unified engine returns a complete safety assessment:

| Field | Type | Description |
| --- | --- | --- |
| risk_level | string | "normal" ¦ "low_risk" ¦ "moderate_risk" ¦ "high_risk" ¦ "critical" |
| risk_score | float | Combined risk score (0.0 – 1.0) |
| reasoning | list[str] | Ordered list of factors that contributed to the risk level |
| recommended_action | string | What the system recommends doing at this risk level |
| alert.title | string | Short, calm alert title for the user |
| alert.message | string | Human-friendly, actionable alert message |
| alert.severity | string | "low" ¦ "medium" ¦ "high" |
| debug.physio_state | string | The underlying physiological state |
| debug.physio_confidence | float | Physiological detection confidence |
| debug.context_score | float | Raw geospatial context risk score |
| debug.geo_available | bool | Whether geospatial context was provided |

## 4.1 Risk Level Definitions

| Level | Score Range | Meaning | System Response |
| --- | --- | --- | --- |
| NORMAL | < 0.20 | No concern. Safe context. | Continue passive monitoring |
| LOW RISK | 0.20 – 0.39 | Mild stress or slightly unusual context | Log context for pattern analysis |
| MODERATE | 0.40 – 0.59 | Stress in concerning context, or distress in safe context | Suggest user check in; enable emergency contacts |
| HIGH RISK | 0.60 – 0.79 | Distress in concerning context | Prompt user to confirm safety; prepare to notify contacts |
| CRITICAL | ≥ 0.80 | Multiple danger signals across physiological and context | Initiate safety protocol; auto-notify if no response in 60s |

# 5. Risk Scoring System

## 5.1 Core Formula

Risk Score Formula risk_score = physio_base × max(0.3, confidence) × context_amplifier Where context_amplifier = 0.8 + (context_score × 0.7) Range: 0.8x (safe context) to 1.5x (dangerous context)

## 5.2 Physiological Base Risk

| Physio State | Base Score | Rationale |
| --- | --- | --- |
| Normal | 0.05 | Very low base — body is fine, context alone shouldn’t alarm |
| Stress | 0.35 | Moderate base — body is showing concern, context determines severity |
| Distress | 0.70 | High base — body is in distress, context can escalate to critical |

The base score is multiplied by max(0.3, confidence) to prevent very low-confidence detections from producing high risk scores. Even if the distress engine outputs "distress" with 0.1 confidence, the effective base is 0.70 × 0.3 = 0.21 rather than 0.07.

## 5.3 Context Dimensions and Weights

The context score is a weighted combination of five independent dimensions, each scored 0–1:

| Dimension | Weight | What It Measures | Key Thresholds |
| --- | --- | --- | --- |
| Time of Day | 25% | How risky the current hour is | Late night (23–04): 0.9 ¦ Night (20–22): 0.6 ¦ Dawn (05–06): 0.4 ¦ Day: 0.1 |
| Location | 30% | Zone familiarity + distance from home | Unfamiliar: 0.7 ¦ Known: 0.15 ¦ Work: 0.05 ¦ Home: 0.0 ¦ Distance adds 0–0.24 |
| Movement | 25% | Movement anomalies and patterns | Route change: +0.5 ¦ Sudden stop: +0.4 ¦ Vehicle at night: +0.2 ¦ High speed: +0.3 |
| Connectivity | 10% | Phone connection status | Disconnected: 0.8 ¦ Connected: 0.0 |
| Familiarity | 10% | Secondary unfamiliarity check | Unfamiliar + not known: 0.6 ¦ Otherwise: 0.0 |

# 6. Time Risk Scoring

Time of day is one of the strongest contextual signals. Crime statistics and safety research consistently show that late-night hours carry disproportionate risk, particularly for women’s safety scenarios.

| Time Window | Hours | Score | Rationale |
| --- | --- | --- | --- |
| Late Night | 23:00 – 04:59 | 0.9 | Peak risk hours. Fewer people around, reduced visibility, limited help access |
| Night | 20:00 – 22:59 | 0.6 | Elevated risk. Darkness, reduced foot traffic |
| Early Morning | 05:00 – 06:59 | 0.4 | Transitional period. Dawn, some risk remains |
| Daytime | 07:00 – 19:59 | 0.1 | Lowest risk. Public activity, visibility, help readily available |

# 7. Location & Familiarity Scoring

## 7.1 Zone Classification

| Zone | Score | How It’s Determined |
| --- | --- | --- |
| Home Zone | 0.0 | User-defined geofence around home address (typically 100–500m radius) |
| Work Zone | 0.05 | User-defined geofence around workplace |
| Known Area | 0.15 | Area visited 3+ times in past 30 days (automatically learned) |
| Unfamiliar Area | 0.70 | No prior visits detected. First-time location. |

## 7.2 Distance Risk Tiers

Distance from home adds an incremental risk contribution on top of zone classification:

| Distance from Home | Additive Score | Effective Contribution |
| --- | --- | --- |
| ≥ 50 km | +0.24 (0.8 × 0.3) | Far from home, unfamiliar territory |
| ≥ 20 km | +0.18 (0.6 × 0.3) | Significantly away from home |
| ≥ 5 km | +0.09 (0.3 × 0.3) | Moderately away from home |
| < 5 km | +0.0 | Close to home, minimal distance risk |

# 8. Movement Risk Scoring

Movement anomalies are strong indicators of concerning situations. These signals are additive — multiple anomalies compound:

| Signal | Score | Detection Method | Safety Rationale |
| --- | --- | --- | --- |
| Sudden Route Change | +0.5 | Heading changed > 90° from expected path | Forced detour, abduction scenario, getting lost |
| Sudden Stop | +0.4 | Speed dropped from > 20 to < 2 km/h rapidly | Vehicle forced to stop, breakdown in unsafe area |
| Vehicle at Night | +0.2 | Vehicle-like speed (> 15 km/h) during night hours | Higher risk of road incidents, unfamiliar drivers |
| High Speed | +0.3 | Speed exceeds 120 km/h | Erratic/reckless driving, potential danger |

Maximum movement score: 1.0 (capped). A sudden route change + sudden stop already scores 0.9.

# 9. Suppression Rules (False Positive Reduction)

Suppression rules reduce false positives by recognizing safe contexts where elevated physiological signals are expected or non-threatening. These rules ONLY apply when physio_state is "stress" — distress is never suppressed.

Important Suppression NEVER applies to distress. If the body shows multi-signal distress, it always receives at least moderate risk regardless of context.

| Rule | Multiplier | Conditions | Example Scenario |
| --- | --- | --- | --- |
| Home Safe | ×0.5 | Home zone + daytime + no movement anomaly | Watching a thriller at home, HR spikes → risk halved |
| Work Safe | ×0.6 | Work zone + business hours (07–20) + no sudden stop | Stressful meeting at the office → risk reduced 40% |
| Known Active | ×0.7 | Known area + walking + daytime + no route change | Working out at the gym, HR elevated → risk reduced 30% |

Example walkthrough:

- User is at the gym (known area, walking, daytime). HR = 110 bpm → stress state.

- Base risk: 0.35 × 0.45 (confidence) = 0.158

- Context amplifier: ~0.84 (low context risk) → 0.158 × 0.84 = 0.133

- Suppression: ×0.7 → 0.133 × 0.7 = 0.093

- Final risk: 0.093 → NORMAL. No alert shown.

# 10. Escalation Patterns

Escalation patterns detect specific dangerous combinations that deserve higher risk than the formula alone would produce. These force a minimum risk score regardless of the computed value.

## 10.1 Critical Patterns (≥ 0.85)

| Pattern | Min Score | Signals Required |
| --- | --- | --- |
| Distress + Night + Unfamiliar + No Phone | 0.90 | Physiological distress in an unknown area at night with lost phone connection |
| Distress + Sudden Stop + Unfamiliar | 0.85 | Body in distress, vehicle suddenly stopped in unfamiliar territory |

## 10.2 High Risk Patterns (≥ 0.65)

| Pattern | Min Score | Signals Required |
| --- | --- | --- |
| Distress + Night + Unfamiliar | 0.75 | Distress at night in an area never visited before |
| Distress + Route Deviation | 0.70 | Distress combined with unexpected route change |
| Vehicle Sudden Stop + Physio Concern | 0.65 | Stress or distress with vehicle coming to sudden halt |

## 10.3 Elevated Patterns (≥ 0.50)

| Pattern | Min Score | Signals Required |
| --- | --- | --- |
| Stress + Night + Unfamiliar + Far | 0.55 | Stress at night, unfamiliar, > 10 km from home |
| Phone Lost + Unfamiliar + Night | 0.50 | Phone disconnected in unknown area at night |

# 11. Alert System

All alerts follow the STAR principle — calm, clear, actionable, and privacy-respecting. The system never uses medical jargon, technical terms, or alarming language.

## 11.1 Alert Design Principles

| Principle | Good Example | Bad Example |
| --- | --- | --- |
| Human language | "We noticed signs of stress in an unfamiliar setting." | "Geospatial anomaly detected in Zone 4." |
| Actionable | "Consider contacting someone you trust." | "Risk level elevated to HIGH." |
| Calm tone | "Are you okay?" | "DANGER: Multiple warning signals!" |
| Privacy-respecting | "Your surroundings seem unusual." | "You are at 17.385°N, 78.486°E, an unfamiliar location." |
| Non-judgmental | "We’re here if you need us." | "You should not be in this area at this time." |

## 11.2 Alert Examples by Risk Level

| Risk Level | Alert Title | Alert Message |
| --- | --- | --- |
| NORMAL | All clear | You seem to be in a familiar place and everything looks fine. We’re here if you need us. |
| LOW RISK | Keeping an eye out | Things look mostly fine, but we noticed a small change. We’ll keep monitoring quietly. |
| MODERATE | Stay aware | We’re picking up some signs worth noting. If you feel uneasy, consider reaching out to someone you trust. |
| HIGH RISK | We’re concerned | We noticed signs of distress in an unfamiliar setting. Are you okay? Consider contacting someone you trust. |
| CRITICAL | Emergency — are you safe? | Multiple warning signs detected. If you’re in danger, please call for help. We can alert your trusted contacts. |

## 11.3 Context-Specific Alerts

The system picks specialized alerts for specific patterns:

- Route deviation: "We noticed you’ve deviated from your usual path. If everything’s fine, no action needed — we’re just keeping watch."

- Phone disconnected: "We’ve lost connection to your phone. If you’re in an unfamiliar area, try to stay in well-lit, public spaces."

- Fall confirmed: "A fall has been detected and you haven’t moved. If you need help, please call out or press the alert button."

# 12. Example Scenarios

## 12.1 Normal at Home — Daytime

| Input | Value |
| --- | --- |
| Physiological | HR=72, SpO2=98.5%, state=NORMAL |
| Context | Home zone, 14:00, phone connected, stationary |
| Context Score | 0.038 |
| Risk Score | 0.038 |
| Risk Level | NORMAL |
| Alert | "You seem to be in a familiar place and everything looks fine." |

## 12.2 Stressed at Gym (Suppressed)

| Input | Value |
| --- | --- |
| Physiological | HR=110, SpO2=97%, state=STRESS |
| Context | Known area, 10:00, walking, phone connected, 1.5 km from home |
| Suppression | Known area + walking + daytime → ×0.7 |
| Risk Score | 0.097 |
| Risk Level | NORMAL (suppressed from low_risk) |
| Reasoning | stress state detected → suppressed: known area, walking, daytime |

## 12.3 Distress at Night in Unfamiliar Area

| Input | Value |
| --- | --- |
| Physiological | HR=130, SpO2=91%, state=DISTRESS, signals=[hr, spo2] |
| Context | Unfamiliar area, 23:00, stationary, 15 km from home, phone connected |
| Escalation | Distress + night + unfamiliar → force 0.75 |
| Risk Score | 0.75 |
| Risk Level | HIGH RISK |
| Alert | "We noticed signs of distress in an unfamiliar setting. Are you okay?" |
| Action | Prompt user to confirm safety. Prepare to notify trusted contacts. |

## 12.4 Vehicle Sudden Stop at Night

| Input | Value |
| --- | --- |
| Physiological | HR=125, SpO2=93%, state=DISTRESS, signals=[hr, spo2] |
| Context | Unfamiliar, 01:00, vehicle, sudden stop + route change, 25 km from home |
| Escalation | Distress + sudden stop + unfamiliar → force 0.85 |
| Risk Score | 0.85 |
| Risk Level | HIGH RISK (would escalate to CRITICAL after 30s persistence) |
| Reasoning | distress + late night + unfamiliar + route deviation + sudden stop + vehicle |

## 12.5 Critical — Worst Case

| Input | Value |
| --- | --- |
| Physiological | HR=140, SpO2=89%, state=DISTRESS, signals=[hr, spo2] |
| Context | Unfamiliar, 02:00, phone disconnected, 50 km from home, stationary |
| Escalation | Distress + night + unfamiliar + no phone → force 0.90 |
| Risk Score | 0.90 |
| Risk Level | HIGH RISK → CRITICAL (after 30s persistence) |
| Alert | "Multiple warning signs detected. If you’re in danger, please call for help." |
| Action | Initiate safety protocol. Notify emergency contacts if no response within 60s. |

# 13. Anti-Flickering & Persistence

To prevent rapid oscillation between risk levels (which would cause repeated alerts and user fatigue), the system implements persistence requirements:

- Critical persistence: The risk must stay at critical level for 30 consecutive seconds before the system outputs "critical". Until then, it caps at "high_risk".

- Escalation cooldown: Once the high-risk timer resets (risk drops below high), it starts fresh on the next occurrence.

- History tracking: The engine maintains a 120-frame rolling history (~2 minutes at 1 Hz) for state distribution analysis.

Why 30 Seconds? 30 seconds is long enough to filter out transient GPS glitches, momentary sensor noise, or brief stops at traffic lights — but short enough that a real emergency still triggers quickly. This value is configurable.

# 14. Privacy-First Design

Privacy is a core architectural constraint, not an afterthought. For a women’s safety product, the device must protect without surveilling.

## 14.1 Data Flow

| Data | Stored? | Transmitted? | Purpose |
| --- | --- | --- | --- |
| Raw GPS (lat/lon) | NEVER | Never | Used once for distance calc, then discarded |
| Zone classification | Yes (local) | Only in alerts | is_home, is_work, is_known, is_unfamiliar |
| Distance from home | Session only | Never | Computed per-frame, not persisted |
| Time of day | Session only | Never | Used for time risk scoring |
| Movement flags | Session only | Only in alerts | sudden_stop, route_change booleans |
| Risk level | Yes (local) | In alerts only | For history and pattern analysis |
| Reasoning chain | Session only | In alerts only | For user transparency |

## 14.2 Privacy Principles

- Raw coordinates are processed in-memory and immediately discarded after zone classification

- No location history is built or stored — each frame is independent

- Alert messages never include coordinates, addresses, or identifiable locations

- Zone learning ("known area" detection) uses hashed grid cells, not raw GPS

- All processing happens on-device — no cloud dependency for risk assessment

- The user can disable geospatial context entirely; the system falls back to physio-only assessment

# 15. Threshold Tuning Strategy

All thresholds are configurable constants at the top of the module. The recommended tuning approach:

## 15.1 Tuning Parameters

| Parameter | Current Value | Tuning Guidance |
| --- | --- | --- |
| GEO_WEIGHTS | time:0.25, loc:0.30, move:0.25, conn:0.10, fam:0.10 | Increase movement weight if route deviations are strong signals in your user base |
| TIME_RISK | late_night:0.9, night:0.6, dawn:0.4, day:0.1 | Adjust based on local crime statistics and user feedback |
| RISK_THRESHOLDS | critical:0.80, high:0.60, mod:0.40, low:0.20 | Lower thresholds for more sensitive detection; raise for fewer false positives |
| Suppression multipliers | home:×0.5, work:×0.6, known:×0.7 | Increase multipliers (closer to 1.0) for less suppression |
| Critical persistence | 30 seconds | Reduce for faster critical alerts; increase for fewer false criticals |

## 15.2 Calibration Process

- Collect 2–4 weeks of real-world data from beta users with geospatial context

- Identify false positive scenarios (alerts in safe contexts) and false negatives (missed real concerns)

- Adjust suppression multipliers to reduce false positives

- Adjust escalation pattern thresholds to catch false negatives

- A/B test with user cohorts to measure alert fatigue and perceived safety

- Iterate monthly based on user feedback and incident reports

# 16. Implementation Reference

## 16.1 Key Classes

| Class | Purpose | Location |
| --- | --- | --- |
| GeoContext | Typed dataclass for geospatial input with safe defaults | distress_engine.py — Section 10 |
| SafetyRiskEngine | Scores context, combines with physiology, applies suppression/escalation | distress_engine.py — Section 11 |
| UnifiedSafetyEngine | Top-level orchestrator combining all 3 detection layers | distress_engine.py — Section 12 |
| DistressEngine | Physiological distress detection (pre-existing) | distress_engine.py — Section 7 |
| FallDetector | 3-stage fall detection state machine | distress_engine.py — Section 5 |

## 16.2 Key Functions

| Function | Purpose |
| --- | --- |
| compute_context_score(ctx) | Scores all 5 context dimensions, returns (score, reasons) |
| _score_time_risk(ctx) | Scores time-of-day risk (0–1) |
| _score_location_risk(ctx) | Scores zone familiarity + distance risk (0–1) |
| _score_movement_risk(ctx) | Scores movement anomalies (0–1) |
| _compute_combined_risk() | Combines physio base with context amplifier |
| _apply_suppression() | Reduces risk in safe contexts (home, gym, work) |
| _apply_escalation() | Forces minimum scores for dangerous pattern matches |

## 16.3 Usage Example

Python usage for real-time evaluation:

Code Example engine = UnifiedSafetyEngine() for frame in sensor_stream: pipeline_out = preprocessor.step(frame) geo_ctx = phone.get_context() # from companion app result = engine.evaluate(pipeline_out, geo_context=geo_ctx) if result["safety"]["risk_level"] in ("high_risk", "critical"): show_alert(result["alert"]) notify_contacts_if_needed(result["safety"])

End of Document

WUALT Safety Intelligence Engine — Confidential